TLDR on the columns:

| column | meaning |
| --- | --- |
| dist_pith_mm | pith → inner edge of stain crescent (inner radius) |
| dist_edge_mm | stain outer edge → bark, using area radius (r_area − outer) |
| arc_deg | angular spread of crescent around pith |
| arc_len_mm | arc length = arc angle × mean radius |
| max_thick_mm | max radial thickness of crescent |
| area_px | stain area at T10, pixel count |
| area_mm2 | same area in mm² (×0.25²) |
| area_frac | stain area ÷ disc interior area |
| r_area_mm | disc radius from wood area (area-equivalent) |
| r_bearing_mm | disc radius pith→bark along stain bearing (noisy/experimental) |
| dist_edge_bearing_mm | stain outer edge → bark along bearing (noisy) |
| area_t10_mm2 … t100_mm2 | stain area (mm²) at colour thresholds 10…100; T10 = tightest/darkest match, looser as T rises, escapes into normal wood above ~50–60 |

Separately — the overlays: 6 of 7 rendered, the 7th (9.4 m, 23732) got OOM-killed. Fixing memory and finishing it now.

# 1. one-time: install the libraries

pip install numpy pandas pillow scipy matplotlib

# 2. run in this order (metrics first — the others use its CSVs)

python disc_overlay.py        # review images, set bad flags in disc_marks.csv

python disc_band_metrics.py   # metrics, now respecting your flags

python disc_height_charts.py  # charts + wide CSV